Here's a clear, structured, and concise summary of your Electronic Checklist Panel build, including functionality, electronics chosen, reasoning behind each choice, and an estimated cost:

🟢 Project Summary: Electronic Checklist Panel

✅ Functional Overview:

This device is intended as an electronic checklist unit primarily for aviation or similar safety-critical applications. Its primary functionality includes:

- Take-off and Landing Checklists:

  - 8 toggle switches clearly marking completion of checklist items.

  - 16 programmable RGB LEDs (two per switch) indicating the checklist status (white for activated, green for completed).

- Rotary Encoders:

  - Two push-button rotary encoders providing user interaction (parameter adjustments, navigation between screens).

- Sharp Memory LCD Display:

  - Central display (30mm × 40mm) readable in full sunlight.

  - Display screens include:

    - Clock (Local time, UTC, stopwatch)

    - Attitude indicator (similar to Garmin G5)

    - Altitude/pressure/rate of climb/descent

    - Parameter setup screen (UTC/local time, calibration)

- Sensors Integration:

  - IMU (ICM-20948) for orientation (pitch, roll, yaw)

  - Bosch BMP390 (barometer for altitude, pressure)

  - LuminOx AO2 Oxygen sensor (digital UART communication)

  - A MICS-4514 for CO and Nox monitoring

- Power Management & Battery:

  - Powered by two rechargeable Li-ion batteries (2×18650, 7.4V nominal).

  - Charged through USB-C connector.

  - Provides regulated voltages: 8.4V (charging), 5V, 3.3V, and 1.8V rails.

✅ Electronics Selected & Rationale:

| Component | Selection | Reason for Choice |
| --- | --- | --- |
| MCU | STM32L476RGT6 | Low-power, sufficient processing, integrated peripherals |
| Display | ST7789V2 | Sunlight-readable, ultra-low power, excellent outdoor visibility |
| IMU Sensor | ICM-20948 | Precise 9-axis data (gyro, accel, mag), SPI communication |
| Pressure Sensor | Bosch BMP390 | Accurate altitude & pressure, SPI, low power consumption |
| Oxygen Sensor | LuminOx AO2 (UART) | Digital accuracy, robust, simple UART integration |
| CO Sensor | Amphenol MiCS-5524 | Compact, accurate analog measurement |
| Battery Charger | BQ25887 (2S Li-ion) | Safely charges two Li-ion cells from USB-C 5V source |
| Battery Protection | DW01A or similar PCM | Ensures battery safety (overcharge, discharge protection) |
| Buck Regulator (3.3V) | MP1584EN | Efficient, stable 3.3V supply, suitable for battery power |
| LDO (1.8V) | MCP1700-1802 | Precise, low-noise regulator (critical IMU I/O voltage) |
| Rotary Encoders | PEC12R-4130F-S0012 | Reliable, widely available, push-button included |
| Toggle Switches | Miniature SPDT switches | Reliable physical state indication |
| LED Indicators | WS2812B RGB LEDs | Programmable RGB, chainable, clear status indication. 2020 for toggle switch indication and 2 x 5050 for general light |
| Battery Cells | 2×18650 Li-ion cells | Standard, rechargeable, high capacity |

✅ Current Build Status (Summary):

| Stage | Status | Remarks |
| --- | --- | --- |
| Concept & Requirements | ✅ Completed | Functional scope clearly defined |
| Schematic Design | ✅ Completed | Reviewed and corrected against datasheets |
| Sensor Integration Review | ✅ Completed | Datasheets reviewed, corrections applied |
| Power Circuit Design | ✅ Completed | Reviewed and revised for 2-cell charging |
| MCU Selection | ✅ Completed | STM32L476RGT6 reviewed and confirmed |
| GPIO Pin Assignments | ✅ Completed | Validated by datasheet |
| User Interface Design | ✅ Completed | Switches, Encoders, LEDs, LCD finalized |
| PCB Layout | ⏳ Next Step | Ready to begin PCB layout in KiCad |
| Firmware Development | 🕒 Pending | Following PCB layout completion |

✅ Next Steps:

- Begin and complete PCB layout in KiCad.

- Generate and order PCB (Gerber files to JLCPCB).

- Begin firmware development and system integration.

Let me know if you need further clarification or assistance to proceed to PCB layout or firmware stage!